HF -Gen. 15 kW Aircooled

Description DHA 150 (dielectric)

The HF-generator DHA 150 has a rated output of max. 15 kW.

The generator is enclosed in a aluminiumcabinet.  Two sides of the generator are fitted with removable panels.

Controls, instruments, mainsconnections are placed on the front of the generator, and the HF-terminal on the opposite side.

One side of the generator are fitted with three airfilters.

The heated air are distributed out throw the roof and throw the HF-terminal.

The air throw the HF-terminal decreases the condens in the electrode system.

The generator is equipped with power- and gridcontrol to adjust the generator when changing electrodes.  It is also equipped with a separate control box.

Technical data: Electrical connection: as specified 220-50OV 3 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 30 kVA.

Frequency: app. 13,56 MHz or 27,12 MHz.

Oscillator tube: ITL 12- 1.

Rectifiers: Silicon diodes.

Weight: app. 450 kg.

1. Circuit breaker

2. Stop HE

3. Start HF.

4. Switch mantimerelay.

5. Lamp HF.

6. Lamp ready.

7. Lamp controlvoltage.

8. Lamp air.

9. Anode current.

I 0. Grid current.

I 1. Fuses.

12. Grid control.

13 Power control.

14. Mains connection.

15. Timerelay HF.