_j

IHF-Gen. 15 kW A'Ircooled with furnace for

pre -heating of finger joints.

Description DHA 150 (dielectric)

The HF-generator DHA 150

has a rated output ofmax. 15 kW.

The generator is enclosed in

a aluminiumcabinet.  Two sides

ofthe generator are fitted with

removable panels.

Controls, instruments and

mainsconnections are placed on

the front of the generator, and

the HF-terminal on the opposite side.

One side ofthe generator are fitted with

three airfilters.  The heated air is distributed

out through the roof and through the HF-terininal The airflow through the HF-terininal decreases the condensation in the electrode system.

The generator is equipped with gridcontrol to adjust the gridcurrent for different boards.

Technical data:

Generator: Electrical connection: as specified 220-50OV 3 phase, 50@ or 60Hz.

Power consumption at maximum output power: 30 kVA.

Frequency: app. 27,12 MHz.

Oscillator tube: ITL 12- 1.

Rectifiers:	Silicon diodes.

Weight: app. 450 kg.

Furnace:	capacity, 7 kg wood per minute are heated 60 degrees C.

Board ends from 20 - 130mm Lenght of electrode: 1700 mm.

Weight:	app. 120 kg.