Hardening equipment for hydraulic piston.

Description HB 600 B*

The HF-generator IHA 600 has a rated output of max. 60 kW.  'Ihe generator is enclosed in a aluminiumcabinet.  Two sides of the generator are fitted with removable panels.

Controls, insftments and

the HF-terrninal are placed on the front of the generator, and the mains and waterconnecfion on the opposite side.

The output power is continous regulated from I 0 to I 00%.

The generator is equipped with a overload circuit breaker to avoid overloading.

Technical data:

Generator: Electrical connection: as specified 220-50OV 3 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 120 kVA.

Frequency: app. 300 kHz.

Oscillator tube: YD 1202.

Rectifiers: Silicon diodes.

Water consumption: app. 30 I/min.

Weight: app. 1250 kg.

Hardeningmachine:

PLC controlled servomotor, lenght of stroke 200 mm.

Rotation ofpiston max. 600 rpm.

All parameters are set on a digital terminal.

Weight:	app. 15 0 kg.