Nobody can hear your
baby’s pulse . . .

. . .unless you are using
AnJo:s Algorithm

Fetal ECG Signal Extraction

May 2000

Anders Bilde, ets99abi
Jonny Lundin, ets99jlu

# Preface

This report shows a simple example of how to separate a desired signal from several other signals. The study was made at the University of Karlskrona/Ronneby in the Master of Science course Adaptive Signal Processing in the spring of year 2000.

The report is directed to those who have an interest in adaptive algorithms and Wiener filters, but anybody who’s interested in signal processing can also enjoy the material.

Enjoy!

Anders Bilde, ets99abi@student.hk-r.se
Jonny Lundin, ets99jlu@student.hk-r.se
University of Karlskrona/Ronneby, May 2000

# Abstract

To separate one signal from another is one of the main parts in signal processing and adaptive filters are often used. This report will describe how to separate a signal represented by a fetals (unborn child) pulse. The mothers pulse and noise from the measurement equipment disturbs the pulse. With adaptive algorithms the fetal pulse is detected and analysed. The project will compare four different adaptive algorithms to see which one that gets the best fetal pulse. The algorithms which are studied are the LMS, the Leaky LMS (L-LMS), normalised LMS (NLMS) and finally the recursive least squares (RLS).

A Wiener filter over all measure data is used as a reference signal.

The LMS algorithm is very easy. It doesn’t cost many operations. It suppresses the mother’s signal well and the mean-square error  is also quite good and the noise level is very low

The L-LMS algorithm is very much alike the LMS algorithm. It suppresses the mother’s signal even more and  is a little bit better than with the LMS. This is the best algorithm in this project. The number of calculations is a little bit more than with the LMS algorithm.

With NLMS the mother’s signal is not as much suppressed as with the LMS or L-LMS algorithm.  is rather noisy and doesn’t converge to such a good value as with the LMS or L-LMS algorithms. It requires a few additional computations more than the LMS does.

The RLS algorithm is very fast. It suppresses the mothers pulse almost immediately but the output signal is a little bit noisy.  is not as good as with L-LMS. This algorithm also requires a lot of calculations.

Finally all three signals from the mother’s chest are weight together with the best algorithm, the L-LMS algorithm to detect the fetal pulse. With a good reference signal it is now very easy to detect the fetal pulse.

# Contents

Preface	2

Abstract	3

Contents	4

Image contents	4

1 Introduction	5

2 Problem formulation	6

3 Wiener filter	7

4 LMS	8

5 L-LMS (Leaky LMS)	10

6 N-LMS (Normalised LMS)	11

7 RLS (Recursive Least Squares)	12

8 Convergence of the Algorithms	14

9 Weighted L-LMS	15

10 Result	18

11 Appendix A, Matlab code	19

MAIN	19

Wiener filter	20

LMS estimation	20

L-LMS estimation	21

N-LMS estimation	21

RLS estimation	22

Leaky LMS estimation multi	22

12 References	23

13 Index	24

# Image contents

Figure 1, Problem formulation.	6

Figure 2, Flowgraph Wiener filter.	7

Figure 3, Result from the Wiener filter.	7

Figure 4, Adaptive filter.	8

Figure 5, Result from the LMS algorithm.	9

Figure 6, Result from the L-LMS algorithm.	10

Figure 7, Result from the N-LMS algorithm.	11

Figure 8, Result from the RLS algorithm.	13

Figure 9, Result of the mean-square error, , with all the algorithms.	14

Figure 10, Result from the LMS, L-LMS, N-LMS and RLS algorithms.	15

Figure 11, Flowgraph weighted L-LMS signal.	16

Figure 12, Result of all five abdominal signals (X2 - X6) with the L-LMS algorithm.	17

# Introduction

In the medical care you often find electronic equipment that simplifies the doctors work significantly. One area is electrocardiogram that measures the signal generated by the heart, which is measured from the body surface. When ECG is used to measure the heart rate to a foetus a number of problems will appear. At first you have the mothers heart pulse that is much stronger than the fetus (unborn child). Second, when measuring the signal external direct on the mothers’ body, it will cause a lot of disturbance on the measure equipment. The old method to measure the fetus heart beats per minute internal, caused inconvenience for both mother and fetus.

When using the external measure methods the result can be achieved pretty good by using adaptive algorithms to distract the mother’s heartbeats by having a second measure point close to the mother’s heart.

To distract one signal from another is one of the most important problems in signal processing. The desired signal is often disturbed by noise or by other signals. In many cases the original signal is not available, whether it is speech, radar, EEG or image. It can also be disturbed by noise from the source to the measure point.

In very simple and idealised cases the signal can be detected by a low-pass, high-pass or band-pass filter, but it is very rare this filters gives the optimum estimate of the signal. Therefore is often optimum digital filter used such as the digital Wiener-filter.

# Problem formulation

When using external monitoring method the fetal ECG signal can’t be directly sensed. The ECG signal is taken from the mother’s abdominal area, which gives the ECG signal of both mother and fetal and additive noise from the environment around the mother. This is why adaptive algorithms must be used. We have the opportunity to use a reference signal from the mother’s heart.

A sound recording containing eight different sounds has been received and studied. The first five signals contained samples from the abdominal part of the mother and the fetal. The three last signals contained sound samples from the heart area of the mother.

Figure 1, Problem formulation.

# Wiener filter

Figure 2, Flowgraph Wiener filter.

An application of the Wiener filter is noise cancellation. As the x(n) = d(n) + v1(n) where d(n) is the fetal and the v1(n) is the mothers signal from the heartbeat measured at the abdominal part on the mothers skin. The v2(n) is the mothers heartbeat measured at chest. To estimate the v1(n) signal you can se the area between the mothers chest and abdominal area as a transfer function. This estimation requires that the autocorrelation of the heartbeat v(n) is known. It also requires that the signals are wide sense stationary, as presumed in this case. To solve the Wiener filter, the Wiener-Hopf equations was used, Wopt =  Rv2-1 * rv1v2 .

Figure 3, Result from the Wiener filter.

The result from the Wiener filter shows the estimation of the fetal heartbeat ( e(n)=x(n)-v1(n) ). The visual review from the result is that only the fetal signal shows. This result will be the best, because the autocorrelation is using all the samples. This signal is therefore used as a reference signal.

# LMS

Figure 4, Adaptive filter.

The LMS algorithm is developed from the steepest descent algorithm, with the special case of one point sample mean.

wn+1=wn+μ⋅e[n]⋅x[n]

The LMS algorithm doesn’t need many operations to bee computed. An LMS filter having p+1 coefficients requires p+1 multiplication’s and p+1 additions to update the filter coefficients. One addition is also required to calculate the error e[n] = d[n] – y[n] and one multiplication is needed to calculate the product ∙e[n]. Finally p+1 multiplication’s are necessary to calculate the output y[n].

Thus a total of 2∙p+3 multiplications and 2∙p+2 additions per output point are required.

Though the LMS algorithm is rather simple it gives often a very good results.

Figure 5, Result from the LMS algorithm.

The LMS algorithm drops the mother’s signal far under the fetal signal. The mean-square error  is also quite good and the noise level is very low.

# L-LMS (Leaky LMS)

If the input process has an autocorrelation matrix with eigenvalues that are zero or close to zero it will not converge. The solution of the problem is to include a leakage to the algorithm. The effect will be that the weights will be forced to zero if either the error or the input becomes zero, and force any undamped modes of the system to zero.

The L-LMS algorithm shows below.

wn+1=(1−μγ)wn+μ⋅e[n]⋅x[n]0<γ<<1

The number of calculations is a little bit more than with the LMS algorithm, while the constant (1-∙) wn has to be calculated.

Figure 6, Result from the L-LMS algorithm.

The visual conclusion of the graph, comparing to the LMS there is no or very small varieties in amplitude of the fetal. The error is a little biased in the L-LMS comparing to the LMS.

# N-LMS (Normalised LMS)

The implementation and design of LMS there is often a problem to decide what step sizes that is suitable for the algorithm, because the Rx is not always given. Then Rx must be estimated which isn't easy in the most cases. To get around this problem a new variable  is replacing the variable .  is a normalised step size where 0 <  < 2 which gives the Normalised LMS.

μ[n]=βx[n]2

wn+1=wn+β⋅x[n]x[n]2⋅e[n]

When x[n] is large the LMS algorithm a problem with gradient noise amplification. With the normalised LMS the noise problem is dropped. Although when x[n] has a small value the N-LMS algorithm gives a noise signal.

The N-LMS algorithm requires a few additional computations more than LMS to calculate the term ||x[n]||2 above.

Figure 7, Result from the N-LMS algorithm.

The mother’s signal is not as much dropped as with the LMS or L-LMS algorithm. The mean-square error  is also rather noisy and doesn’t converge to such a good (small) value as with the LMS and L-LMS algorithms. The long duration before the output signal starts is because the filter length is rather big (150 samples).

# RLS (Recursive Least Squares)

zn=Pn−1⋅x∗ngn=znλ+xTn⋅znαn=dn−wn−1T⋅xnwn=wn−1+αn⋅gnPn=Pn−1−gn⋅zHnλen=dn−xn⋅wnT⋅xn

When [n] is small, the filter coefficient is close to the optimal value and only a small correction is needed to adjust the coefficient. When [n] is big, the filter coefficient are not performing well with d[n] and a large correction has to be done.

The exponential weighting RLS is used. That means that the algorithm is multiplicated by a constant  that makes that it can follow the signal better, even if big jumps appear, but the output signal will be noisier.

The number of operations while using RLS is rather big. When using the LMS algorithm that requires p numbers of algorithms the RLS requires p2 operations. Generally the RLS algorithm converges faster than the LMS algorithm for stationary processes. On the other hand, RLS does not perform very well in tracking nonstationary processes. Although exponential weighting improves the tracking characteristic of RLS, it is not clear how to chose  and in some cases the RLS may have better tracking properties.

The RLS-algorithm is very fast. It suppresses the mothers pulse almost immediately (during the first pulse).

Figure 8, Result from the RLS algorithm.

The RLS algorithm drops the mother’s signal under the fetal signal but not as much as LMS or L-LMS does and the output signal is a noisier. The mean-square error  drops to a lower level much faster than LMS or L-LMS, but it is a little bit noisier.

# Convergence of the Algorithms

The Wiener solutions are we using as the optimal solution. The mean-square error  is computed, as the difference between desired signal e and the Wiener solution signal, y_wiener.

 = 10*log (abs (e – y_wiener)2 )

Figure 9, Result of the mean-square error, , with all the algorithms.

The mean-square error,  of the N-LMS algorithm is very noisy.  of the RLS algorithm is very fast but it is still noisy. The L-LMS algorithm has the best (lowest)  value.

# Weighted L-LMS

To decide which algorithm to implement with, the final weighted reference signal is tested with several different algorithms. The tests of the algorithms was made by using the best signal of the reference signal and the fetal signal so the result easier could be examined. By closer examining the figure below the following conclusions can be made.

Figure 10, Result from the LMS, L-LMS, N-LMS and RLS algorithms.

The LMS result suppresses the maternal signal far below the fetal signal. The Leaky LMS result shows the best result of all algorithms where the fetal signal is at least three times larger than the maternal signal. The N-LMS result was not as good as L-LMS while the noise level was much higher. Even the result of the RLS was noisier than the L-LMS result.

The thoracic signals is weighted together to achieve a more reliable average of the reference signal.

Figure 11, Flowgraph weighted L-LMS signal.

The L-LMS algorithm is used and adds the three signals from the mother’s chest (X7, X8 and X9) and subtracts them from the desired signal from the abdominal signals. This procedure is repeated for all five abdominal signals (X2 - X6).

The results of all five abdominal signals (X2 - X6) are shown in the figure below.

Figure 12, Result of all five abdominal signals (X2 - X6) with the L-LMS algorithm.

In the first signal (e2) you can very easy detect the babies pulse. Even in signal e3 and e4 you can also detect it. Signal e5 has such a big waveform that it is almost impossible to detect the babies pulse from it automatically. Finally in signal e6 the baby’s signal is still hidden in too much noise.

# Result

The LMS algorithm drops the mother’s signal far under the fetal signal. The mean-square error  is also quite good and the noise level is very low. It doesn’t require much operations to compute the mean-square error .

The Leaky LMS algorithm is the best in this project. It drops the mother’s signal most and gives the best SNR in this project. It is a rather fast algorithm and the mean-square error  is also very low. The number of calculations is a little bit more than with the LMS algorithm. This algorithm is used to calculate the SNR with all the other abdominal signals.

The mother’s signal is not as much dropped as with the LMS or L-LMS algorithm. The mean-square error is also rather noisy and doesn’t converge to such a good (small) value as with the LMS and L-LMS algorithms.

The N-LMS algorithm is very noisy and the mother’s signal is not dropped as much as with the LMS or L-LMS algorithms. The mean-square error  is also rather noisy and doesn’t converge to such a good (small) value as with the other algorithms. The number of calculations is a little bit more than with the LMS algorithm. This algorithm gives the worst output data in our project.

The RLS algorithm is a very fast algorithm, but it is not good for big jumps in the signal. Therefore exponential RLS is used, with a constant , which makes that the algorithm follows the desired signal better, but it makes the output signal noisier. This algorithm also requires much more number of operations than the others.

If a signal is bad from the beginning it is very difficult to correct it with these algorithms. Therefore it is very important to have a good contact to the measure object to get such a good signal as possible.

# Appendix A, Matlab code

## MAIN

% Adaptiv Project by Jonny Lundin och Anders Bilde 2000-05-16

%%%%%%%%%%%%%%%%%%%%%%

% INDEXERING			%

%%%%%%%%%%%%%%%%%%%%%%

clear all;

close all;

%load foetal_ecg.dat

load ecg.dat

y=1:2500;

%x1=ecg(y,1);

x2=ecg(y,2);

x3=ecg(y,3);

x4=ecg(y,4);

x5=ecg(y,5);

x6=ecg(y,6);

x7=ecg(y,7);

x8=ecg(y,8);

x9=ecg(y,9);

x7x=x7/300;

x8x=x8/300;

x9x=x9/300;

figur_nr=0;

figur_nr=figur_nr+1;figure(figur_nr);

subplot(5,1,1);plot(x2,'r');title('X2 Child 1');grid;ylabel('Amplitude');

subplot(5,1,2);plot(x3,'r');title('X3 Child 2');grid;ylabel('Amplitude');

subplot(5,1,3);plot(x4,'r');title('X4 Child 3');grid;ylabel('Amplitude');

subplot(5,1,4);plot(x5,'r');title('X5 Child 4');grid;ylabel('Amplitude');

subplot(5,1,5);plot(x6,'r');title('X6 Child 5');grid;ylabel('Amplitude');xlabel('Time(sek)/250 Hz');

figur_nr=figur_nr+1;figure(figur_nr);

subplot(3,1,1);plot(x7,'b');title('X7 Mom 1');grid;ylabel('Amplitude');

subplot(3,1,2);plot(x8,'b');title('X8 Mom 2');grid;ylabel('Amplitude');

subplot(3,1,3);plot(x9,'b');title('X9 Mom 3');grid;ylabel('Amplitude');xlabel('Time(sek)/250 Hz');

%%%%%%%%%%%%%%%%%%%%%%

% Wiener filter		%

%%%%%%%%%%%%%%%%%%%%%%

x=x9/100; % Chest Signal

d=x2; % Abdominal Signal

N=10;	% Filterlängd

[y_w]=wiener(x,d,N);

figur_nr=figur_nr+1;figure(figur_nr);

subplot(3,1,1),plot(x9,'b');title('Mom');grid;ylabel('Amplitude');

subplot(3,1,2),plot(x2,'r');title('Mom + Child');grid;ylabel('Amplitude');

subplot(3,1,3),plot(y_w,'m');title('Wiener filtrerad');grid;ylabel('Amplitude');xlabel('Time(sek)/250 Hz');

%%%%%%%%%%%%%%%%%%%%%%

% LMS estimation 		%

%%%%%%%%%%%%%%%%%%%%%%

my=0.02;

L=10;

[y,e,f]=lms(x9x,x2,my,L);

y_lms=e;

epsilon_lms=10*log10(abs((y_lms-y_w).^2));

figur_nr=figur_nr+1;figure(figur_nr);

subplot(4,1,1),plot(x9,'b');title('Thoracic (chest) signal');grid;ylabel('Amplitude');

subplot(4,1,2),plot(x2,'r');title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(4,1,3),plot(y_lms,'m');zoom on;title('Fetal heart puls with estimated LMS');grid;ylabel('Amplitude');

subplot(4,1,4),plot(epsilon_lms,'g');zoom on;title('Epsilon^2 LMS');grid;axis([0 2500 -50 50]);ylabel('dB');...

xlabel('Iterations');

%%%%%%%%%%%%%%%%%%%%%%

% L-LMS estimation 	%

%%%%%%%%%%%%%%%%%%%%%%

gamma= 0.01;

my=0.02;

L=10;

[y,e,f]=llms(x9x,x2,my,gamma,L);

y_llms=e;

epsilon_llms=10*log10(abs((y_llms-y_w).^2));

figur_nr=figur_nr+1;figure(figur_nr);

subplot(4,1,1),plot(x9,'b');title('Thoracic (chest) signal');grid;ylabel('Amplitude');

subplot(4,1,2),plot(x2,'r');title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(4,1,3),plot(y_llms,'m');title('Fetal heart puls with estimated with L-LMS');grid;ylabel('Amplitude');

subplot(4,1,4),plot(epsilon_llms,'g');zoom on;title('Epsilon^2 L-LMS');grid;axis([0 2500 -50 50]);ylabel('dB');...

xlabel('Iterations');

%%%%%%%%%%%%%%%%%%%%%%

% N-LMS estimation 	%

%%%%%%%%%%%%%%%%%%%%%%

beta=0.5;

L=150;

[y,e,f]=nlms(x9x,x2,beta,L);

y_nlms=e;

epsilon_nlms=10*log10(abs((y_nlms-y_w).^2));

figur_nr=figur_nr+1;figure(figur_nr);

subplot(4,1,1),plot(x9,'b');title('Thoracic (chest) signal');grid;ylabel('Amplitude');

subplot(4,1,2),plot(x2,'r');title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(4,1,3),plot(y_nlms,'m');title('Fetal heart puls with estimated with N-LMS');grid;ylabel('Amplitude');

subplot(4,1,4),plot(epsilon_nlms,'g');zoom on;title('Epsilon^2 N-LMS');grid;axis([0 2500 -50 50]);ylabel('dB');...

xlabel('Iterations');

%%%%%%%%%%%%%%%%%%%%%%

% RLS estimation 		%

%%%%%%%%%%%%%%%%%%%%%%

lambda=0.99;

nord=10;

x=x9x;

d=x2;

[W,Er]=rls(x,d,nord,lambda);

y_rls=Er.';

epsilon_rls=10*log10(abs((y_rls-y_w).^2));

figur_nr=figur_nr+1;figure(figur_nr);

subplot(4,1,1),plot(x9,'b');title('Thoracic (chest) signal');grid;ylabel('Amplitude');

subplot(4,1,2),plot(x2,'r');title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(4,1,3),plot(y_rls,'m');title('Fetal heart puls with estimated with RLS');grid;ylabel('Amplitude');

subplot(4,1,4),plot(epsilon_rls,'g');zoom on;title('Epsilon^2 RLS');grid;axis([0 2500 -50 50]);ylabel('dB');...

xlabel('Iterations');

%%%%%%%%%%%%%

% SUM PLOT 	%

%%%%%%%%%%%%%

%y

figur_nr=figur_nr+1;figure(figur_nr);

subplot(6,1,1),plot(x2,'r');title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(6,1,2),plot(y_w,'m');title('Wiener filtered');grid;ylabel('Amplitude');

subplot(6,1,3),plot(y_lms,'m');title('Fetal heart puls with estimated LMS');grid;ylabel('Amplitude');

subplot(6,1,4),plot(y_llms,'m');title('Fetal heart puls with estimated L-LMS');grid;ylabel('Amplitude');

subplot(6,1,5),plot(y_nlms,'m');title('Fetal heart puls with estimated with N-LMS');grid;ylabel('Amplitude');

subplot(6,1,6),plot(y_rls,'m');title('Fetal heart puls with estimated with RLS');grid;xlabel('Iterations');...

ylabel('Amplitude');

%Epsilon

figur_nr=figur_nr+1;figure(figur_nr);

subplot(6,1,1),plot(x2,'r');zoom on;title('Abdominal (child + mothers) signal');grid;ylabel('Amplitude');

subplot(6,1,2),plot(y_w,'m');zoom on;title('Wiener filtered');grid;ylabel('Amplitude');xlabel('Time Fs=250 Hz');

subplot(6,1,3),plot(epsilon_lms,'g');zoom on;title('Epsilon^2 LMS');grid;axis([0 2500 -50 50]);ylabel('dB');

subplot(6,1,4),plot(epsilon_llms,'g');zoom on;title('Epsilon^2 L-LMS');grid;axis([0 2500 -50 50]);ylabel('dB');

subplot(6,1,5),plot(epsilon_nlms,'g');zoom on;title('Epsilon^2 N-LMS');grid;axis([0 2500 -50 50]);ylabel('dB');

subplot(6,1,6),plot(epsilon_rls,'g');zoom on;title('Epsilon^2 RLS');grid;axis([0 2500 -50 50]);ylabel('dB');...

xlabel('Iterations');

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% Leaky LMS estimation multi	%

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

gamma= 0.01;

my=0.02;

L=10;

X=[x7x x8x x9x];

[y2,e2,f2]=llms_3(X,x2,my,gamma,L);

[y3,e3,f3]=llms_3(X,x3,my,gamma,L);

[y4,e4,f4]=llms_3(X,x4,my,gamma,L);

[y5,e5,f5]=llms_3(X,x5,my,gamma,L);

[y6,e6,f6]=llms_3(X,x6,my,gamma,L);

figur_nr=figur_nr+1;figure(figur_nr);

subplot(6,1,1),plot(x2,'r');title('X2 referens signal');grid;ylabel('Amplitude');

subplot(6,1,2),plot(e2,'m');title('Fetal heart puls with estimated LLMS e2');grid;ylabel('Amplitude');

subplot(6,1,3),plot(e3,'m');title('Fetal heart puls with estimated LLMS e3');grid;ylabel('Amplitude');

subplot(6,1,4),plot(e4,'m');title('Fetal heart puls with estimated LLMS e4');grid;ylabel('Amplitude');

subplot(6,1,5),plot(e5,'m');title('Fetal heart puls with estimated LLMS e5');grid;ylabel('Amplitude');

subplot(6,1,6),plot(e6,'m');title('Fetal heart puls with estimated LLMS e6');grid;ylabel('Amplitude');...

xlabel('Iterations');

## Wiener filter

%  Projekt Adaptiv Wiener filter algoritm av Jonny Lundin och Anders Bilde 2000-05-04

function [y_w]=wiener(x,d,N);

X=convm(x,N);

D=convm(d,N);

X1=X(1:2500,1:N);

Rxx=X1'*X1;

rxd=X1'*d;

Wopt=Rxx\rxd;

y1=conv(x,Wopt);

y=y1(1:2500);

y_w=d-y;

## LMS estimation

%  Projekt Adaptiv LMS algoritm av Jonny Lundin och Anders Bilde 2000-05-04

function [y,e,f]=lms(x,d,my,L);

N=length(x);

f=zeros(L,1);% nollvektor

y=zeros(size(d));

e=zeros(size(d));

for k=L:N;

xn=x(k:-1:k-L+1).';

y(k)=xn*f;

e(k)=d(k)-y(k);

f=f+(my*e(k)*xn)';

end

## L-LMS estimation

%  Projekt Adaptiv LLMS algoritm av Jonny Lundin och Anders Bilde 2000-05-04

function [y,e,f]=lms(x,d,my,gamma,L);

N=length(x);

f=zeros(L,1);% nollvektor

y=zeros(size(d));

e=zeros(size(d));

for k=L:N;

xn=x(k:-1:k-L+1).';

y(k)=xn*f;

e(k)=d(k)-y(k);

f=f*(1-gamma*my)+(my*e(k)*xn)';

end

## N-LMS estimation

%  Adaptiv NLMS algoritm av Jonny Lundin och Anders Bilde 2000-05-11

function [y,e,f]=nlms(x,d,beta,L);

N=length(x);

f=zeros(L,1);% nollvektor

y=zeros(size(d));

e=zeros(size(d));

p=2;

for k=L:N,

xn=x(k:-1:k-L+1).';

y(k)=xn*f;

e(k)=d(k)-y(k);

xnorm=xn*xn.'+eps;

f=f+(beta*e(k)*xn/xnorm)';

end

## RLS estimation

% RLS algoritm av Jonny Lundin och Anders Bilde tagen ur Adaptiv Boken

function [W,Er] = rls(x,d,nord,lambda)

delta = 0.001;

X=convm(x,nord);

[M,N] = size(X);

if nargin < 4, lambda = 1.0;	end

P=eye(N)/delta;

W(1,:)=zeros(1,N);

for k=2:M-nord+1;

z=P*X(k,:)';

g=z/(lambda+X(k,:)*z);

alpha=d(k)-X(k,:)*W(k-1,:).';

W(k,:)=W(k-1,:)+alpha*g.';

P=(P-g*z.')/lambda;

Er(k)=d(k)-X(k,:)*W(k,:).';

end;

## Leaky LMS estimation multi

%  Projekt Adaptiv LLMS_3 algoritm av Jonny Lundin och Anders Bilde 2000-05-04

function [y,e,f]=llms_3(X,d,my,gamma,L);

N=length(X);

f=zeros(L,3);% nollmatris

y=zeros(size(d),3);

e=zeros(size(d));

for k=L:N;

xn=X(k:-1:k-L+1,1:3);%.';

y(k,1:3)=(diag(xn.'*f)).';

y_tot(k)=y(k,1)+y(k,2)+y(k,3);

e(k)=d(k)-y_tot(k);

f=f*(1-gamma*my)+(my*e(k)*xn);

end

# References

Statistical Digital Signal Processing and modeling

Monson H. Hayes

Georgia Institute of Technology

John Wiley & Sons, Inc, 1996

# Index

C

Convergence	13

E

ECG signal	5

Exponential weighting RLS	11

F

Fetus (unborn child)	4

L

L-LMS (Leaky LMS)	9

LMS	7

N

N-LMS (Normalised LMS)	10

R

Result from the L-LMS algorithm.	9

Result from the LMS algorithm.	8

Result from the N-LMS algorithm.	10

Result from the RLS algorithm.	12

Result from the Wiener filter	6

Result of the mean-square error.	13

RLS (Recursive Least Squares)	11

T

Thoracic	15

W

Weighted L-LMS	14

Wiener filter	6

Wiener-Hopf equations	6