Aircooled

Description DHA 250 (dielectric)

The HF-generator DHA 250 has a rated output of max. 25 kW.

The generator is enclosed in a aluminiumcabinet.  Two sides of the generator are fitted with removable panels.

Controls, instruments, mainsconnections are placed on the front of the generator, and the HF-terminal on the opposite side.

One side of the generator are fitted with three airfilters.

The heated air are distributed out throw the roof and throw the HF-tenninal.

The air throw the HF-terminal decreases the condens in the electrode system.

The generator is equipped with power- and gridcontrol to adjust the generator when changing electrodes.  It is also e i ed with a se arate

electrodes.  It is also equipped with a separate

control box.

Technical data: Electrical connection: as specified 220-50OV 3 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 50 kVA.

Frequency: app. 13,56 MHz or 27,12 MHz.

Oscillator tube: ITL 12- 1.

Rectifiers:	Silicon diodes.

Weight: app. 650 kg.

1.	Grid control.

2.	Anode current.

3, ddd current.

4.	Lainp controlvoltage.

5.	Switch mantimerelay.

6. Fuses.

7. Fuses.

8.	Overload circuit

breaker

9. Main switch.