I,-

0         . 1'      Aircooled

Description DHA 400 (dielectric)

The HF-generator DHA 400 has a rated output of max. 40 kW.

The generator consists of 3 units, oscillator-, power- and controlunit.

The oscillator is enclosed in a aluminum cabinet, one side is fitted with a removable panel.  The powerunit is contained in two mounted cabinets, one of them contains rectifier and the highvoltage transformer, the other cabinet contains contactors and supervision units.  The anode and gridcurrent instruments and control switches are placed in the controlunit.

Technical data: Electrical connection: as specified 220-50OV 3 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 70 kVA.

Frequency: app. 13,56 MHz.

Oscillator tube: ITL 15-2.

Rectifiers: Silicon diodes.

Dimensions (mm):	length	height	depth	weight (kg)

oscillator:	860	2000	630	300

powerunit:	2050	2000	450	800

controlbox:	700	500	250	20