Description IHA 150

The HF-generator IHA 150 has a rated output of max. 15 kW.

The generator is enclosed in a aluminiumcabinet.  Two sides of lhe generator are fitted with removable panels.

Controls, instruments, HF-tenninal and signal lamps are placed on the front of the generator, lhe mainsand water-connections on the opposite side.  The output power is continuous regulated from 10 to 100%.

The capacitor-battery contains two water c'ooled capacitors, reversible to be adjusted to heat treatment of aswell magnetic as unmagnetic material.

The generator is equipped with a over load circuit breaker to avoid overloading and its also equipped with a time relay for the HF-time.

Technical data: Electrical connection: as specified 220-50OV 3 phase, 50Hz or 60Hz.

Power consumption at maximum output power: 30 kVA.

Frequency: ca 0,5-0,8 MHz.

Oscillator tube: ITK 12- 1.

Rectifiers: Silicon diodes.                                                        I Up

Weight: ca 450 kg.

Water consumption: 10- 12 I/min.

1. Power control.	8. Start EF.	9

2

2. Anode current.	9. Switch man-

3. Grid current.	timerelay.

4. Lamp power control.	I 0. Timerelay.

5. _Lwnp ready,	I 1. Power output,

6. Lamp BF.	12. Grid control,	71

7. Stop HF.	13. Main switch.